Lab 8 Deliverables

Akaash Chikarmane and Milan Feliciello

Section: 16085

Spring 2016

Schematic

Time Measurements (like Figure 8.6)

ADC execution time

LCD execution time

Calibration Data

| Position (cm) | Analog Input (V) | ADC Sample |
| --- | --- | --- |
| 0 | 0.01 | 13 |
| 0.25 | 0.12 | 150 |
| 0.5 | 0.62 | 700 |
| 0.75 | 1.07 | 1250 |
| 1 | 1.6 | 1900 |
| 1.25 | 2.08 | 2500 |
| 1.5 | 2.53 | 3100 |
| 1.75 | 3.03 | 3750 |
| 2 | 3.28 | 4095 |

Final Distance Meter

void SysTick_Handler(void){

GPIO_PORTF_DATA_R ^= 0x0C;

ADCMail = ADC_In();

ADCStatus = 0x00000001;

GPIO_PORTF_DATA_R ^= 0x0C;

return;

}

uint32_t delay;

// ADC initialization function

// Input: none

// Output: none

void ADC_Init(void){

SYSCTL_RCGC2_R |= 0x10;				//1) activate PORTE clock

while(SYSCTL_RCGC2_R == 0){		//null

}

GPIO_PORTE_DIR_R &= ~0x04;		//2) make PE2 input

GPIO_PORTE_AFSEL_R |= 0x04;		//3) enable alternate function for PE2

GPIO_PORTE_DEN_R &= ~0x04;		//4) disable digital I/O for PE2

GPIO_PORTE_AMSEL_R |= 0x04;		//5) enable analong function on PE2

SYSCTL_RCGCADC_R |= 0x01;			//6) activate ADC0

delay = SYSCTL_RCGCADC_R;			//null

delay = SYSCTL_RCGCADC_R;			//null

delay = SYSCTL_RCGCADC_R;			//null

delay = SYSCTL_RCGCADC_R;			//null

ADC0_PC_R = 0x01;							//7) configure for 125k

ADC0_SSPRI_R &= ~0x0123;			//8) Seq3 is highest priority

ADC0_ACTSS_R &= ~0x0008;			//9) disable sample sequencer 3

ADC0_EMUX_R &= ~0xF000;				//10) seq3 is software trigger

ADC0_SSMUX3_R = (ADC0_SSMUX3_R & 0xFFFFFFF0) + 1;		//11) Ain1 (PE2)

ADC0_SSCTL3_R = 0x0006;				//12) no TS0D0, yes IE0 END0

ADC0_IM_R &= ~0x0008;					//13) disable SS3 interrupts

ADC0_ACTSS_R |= 0x0008;				//14) enable sample sequencer 3

}

//------------ADC_In------------

// Busy-wait Analog to digital conversion

// Input: none

// Output: 12-bit result of ADC conversion

uint32_t ADC_In(void){

uint32_t result;

ADC0_PSSI_R = 0x0008;				//1) initiate SS3

while((ADC0_RIS_R & 0x08) == 0){		//2) wait for conversion to finish

}

result = ADC0_SSFIFO3_R & 0xFFF;	//3) read 12-bit result

ADC0_ISC_R = 0x0008;				//4) acknowledge completion

return(result);

}

uint32_t Convert(uint32_t input){

uint32_t converted;

converted = input*100;

converted = (converted + 29203);

converted = (converted/223);

return(converted);

}

int main(void){

TExaS_Init();         // Bus clock is 80 MHz

ST7735_InitR(INITR_REDTAB);

PortF_Init();

ADC_Init();         // turn on ADC, set channel to 1

SysTick_Init();

while(1){

if(ADCStatus == 1){

ADCStatus = 0;

measurement = Convert(ADCMail);

ST7735_SetCursor(0,0);

LCD_OutFix(measurement);

ST7735_OutChar(0x20);

ST7735_OutChar(0x63);

ST7735_OutChar(0x6D);

}

}

}

Sampling Rate of 40 Hz (Figure 8.8)

Accuracy Data and Accuracy Calculation

| True Values | Measured values |  | Errors |
| --- | --- | --- | --- |
| 0 | 0.136 |  | -0.136 |
| 0.5 | 0.444 |  | 0.056 |
| 1 | 0.982 |  | 0.018 |
| 1.5 | 1.521 |  | -0.021 |
| 2 | 1.967 |  | 0.033 |
| Average error |  |  |  |
| 0.053 |  |